SIT210 – Embedded Systems Development

Task 3.1P – Create Trigger/Notification Based on Sensor Data

Student: Tien Huy Nguyen |  Platform: IFTTT |  Board: Arduino Nano 33 IoT

# 1. System Description

This embedded system monitors the light level (in lux) on a terrarium using a BH1750 digital light sensor connected to an Arduino Nano 33 IoT over the I2C bus. The Arduino triggers the IFTTT Webhooks service via Wi-Fi (using HTTPS) when the light level reaches a threshold (300 lux). IFTTT then sends an email to the terrarium owner's Deakin University email address to notify them the sun has begun to shine or is no longer shining on the terrarium.

## a) Block Diagram

| BH1750 Light Sensor (I2C: SDA/SCL) | → | Arduino Nano 33 IoT (reads lux, checks threshold) | → | Wi-Fi (HTTPS POST) maker.ifttt.com | → | IFTTT Webhooks (sunlight_on/off) | → | 📧 Email Deakin student email |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

## b) Trigger Mechanism

The trigger checks the lux value from the BH1750 sensor every 3 seconds. It maintains a light state variable (sunlightDetected) to determine if sunlight is detected. The trigger is only activated when there is a change in the light state (sunlight starts when above the threshold and sunlight stops when below the threshold). This avoids unnecessary notifications when the state hasn't changed.

Trigger Code (loop function) float lux = lightMeter.readLightIntensity(); // Sunlight started: lux crossed ABOVE threshold if (lux > lightThreshold && !sunlightDetected) { sunlightDetected = true; sendIFTTTEvent(sunlightOnEvent, "Lux=" + String(lux)); } // Sunlight stopped: lux crossed BELOW threshold else if (lux <= lightThreshold && sunlightDetected) { sunlightDetected = false; sendIFTTTEvent(sunlightOffEvent, "Lux=" + String(lux)); } delay(3000); // check every 3 seconds

The lightThreshold is set to 300 lux. The BH1750 sensor returns a 16-bit unsigned integer value of lux. The change detection (currentState vs previousState) is used to send only one trigger per event and not one per loop iteration.

## c) Notification Mechanism

The notification system uses the IFTTT Webhooks service. The Arduino sends an HTTP GET request to maker.ifttt.com (on HTTPS:443) with the event name and lux value as a query parameter when the trigger condition is true. IFTTT then matches this to the applet and sends an email to the student's Deakin account.

Notification Code (sendIFTTTEvent function) void sendIFTTTEvent(String eventName, String value1) { if (!client.connect(IFTTT_HOST, IFTTT_PORT)) { Serial.println("Connection to IFTTT failed"); return; } String url = "/trigger/" + eventName + "/with/key/" + String(IFTTT_KEY) + "?value1=" + value1; client.println("GET " + url + " HTTP/1.1"); client.println("Host: " + String(IFTTT_HOST)); client.println("Connection: close"); client.println(); }

The IFTTT applet is set up with the event name sunlight_on (for sunlight start) and sunlight_off (for sunlight end). The lux reading is transmitted as value1 and shown in the email body as {{Value1}}, so the user can know the value of the light at the time of each event. The IFTTT Email action sends the email to s223009962@deakin.edu.au.

# 2. Testing the System

To test the system, the Arduino is again connected to the computer using USB and the Serial Monitor is set to 9600 baud. Various light conditions are tested with the BH1750 sensor: first by blocking it with a hand, then removing the block and shining a bright light or window light at it to simulate sunlight. The Serial Monitor is checked to ensure that the lux levels are updated and the IFTTT event is triggered when the value is above 300 lux with the response being HTTP 200 OK to indicate that it was successful.

To confirm that the notification was successfully sent, the student's Deakin email inbox is checked to ensure that an email from action@ifttt.com is received with the expected event name (sunlight_on or sunlight_off) and lux value in the body. The sensor is again covered to trigger the sunlight_off event and a second email should be received. A full test cycle will include the two transitions: dark to bright and bright to dark, with one email for each.

# 3. GitHub Repository

The code has been uploaded to the Task3.1Trigger folder in the GitHub repository:

https://github.com/huyng03/SIT210-730/upload/main

# 4. YouTube Video.

# https://youtube.com/shorts/mkRbT7HJWoE